BELL 206 check A and pre-flight inspection

The Check A is to be carried out prior to the first flight of the day and must be certified.  The Pre-flight check need only cover those items marked with an  and need not be certified by an authorised person, but should be signed for as having been completed in the technical log.

Normal procedures

Check A and preflight	Card 2

Interior	Card 7

Pre start	Card 8

Engine Starting	Card 9

Pre take off/After Take-off	Card 12

Departure/Cruise/Joining	Card 12

Over water  flight	Card 12

Shutdown	Card 13

Emergencies

CWP Captions	Card 15

Engine failure in flight	Card 17

Engine failure in the hover	Card 18

Engine fire during start up	Card 18

Engine fire during flight	Card 18

Tail rotor Control failure	Card 19

Loss of tail rotor thrust	Card 19

Fuel control failure	Card 19

Hydraulics failure	Card 20

Electrical failure	Card 20

Fuel system failure	Card 21

Engine air start	Card 21

Limitations	Card 22

# External

	Fire Extinguisher	Available

	Engine blanks	Remove and stow

	Main rotor tie down	Remove and stow

	Ensure aircraft clear of frost, ice or snow

	Battery switch	On

Landing lights, Navigation Lights	Check On then Off

Pitot heat	Check warming, then Off

	Fuel quantity	Check

	Battery switch	Off

Float System check

Battery switch	On

Master caution CB	In

Float manual arm switch	Off and guarded

Float power CB	In

Caution panel	Test, check Float Armed and Test lights illuminate

Float test switch	Test position and hold

Trigger	Pull

Float test switch	Release, Test light Out

Float Manual Arm switch	Guard up, switch On

Float Arm light	On

Float Manual Arm switch	Off and guarded

Battery switch	Off

Starboard Fuselage

	Static port	Clear

	Cabin doors	Condition and security

	Windows	Condition and security

	Landing gear	Condition

Forward and aft crosstube fairings	Secure, Condition and aligned

	Cabin roof, transmission fairing	Clear of debris

Hydraulic reservoir	Check level

Hydraulic system filter	Bypass indicator In

Hydraulic actuators	Check linkages, leaks and condition

	Forward fairing	Secure

	Hydraulic access panel	Secured

	Transmission Oil	Check level

Transmission mounts	Condition and security

Isolation mount	Condition

Drag pin	Condition, no evidence of contact

Main driveshaft forward coupling	Condition, check temperature indicators

In accordance with AD 96-06-04

	Access panel	Secured

	Air induction cowling	Secured

	Engine inlet and Plenum	Clear of debris

	Fuel cap	Secure

Starboard engine bay

Engine	Condition, security of attachments and oil leaks

Main driveshaft coupling	Condition, check temperature indicators

In accordance with AD 96-06-04

Engine mounts	Condition and Security

Throttle linkage	Condition

Fuel control and governor	Evidence of leakage

Hoses and tubing	Chafing, security and condition

Exhaust stack and clamp	Security and condition

Engine cowl	Secure

	Generator cooling scoop	Clear of debris

	Right exhaust cover	Removed

	Oil tank	Check level

Caution:  The engine oil tank sight gauge mounted on the left side is no  indicative of actual oil quantity.  Oil quantity shall be checked with dipstick.

	Access panel	Secured

	Aft fairing	Secure

	Fuselage	Condition

	Tail rotor driveshaft cover	Condition and secured

	Tailboom	Condition

Position light	Condition and security

	Horizontal stabiliser	Condition and secured

	Main rotor blade	Condition

	Vertical fin	Condition

	Tail rotor guard	Condition and Security

Anti collision light	Condition

Aft position light	Condition

	Tail rotor gearbox	Security, oil level, no leaks

	Tail rotor	Condition and free movement

Tail rotor controls	Condition and security

	Tail rotor blades	Condition. check tip block for rivet damage, corrosion and seal

Port fuselage

	Main rotor blade	Condition

	Horizontal stabiliser	Condition and security

Position light	Condition and security

	Tailboom	Condition

	Tail rotor driveshaft cover	Condition and secured

	Fuselage	Condition

Forward tail rotor driveshaft	Condition of splined adapter and freedom of rotation of witness pin

Oil cooler blower shaft bearings	No grease leakage

Oil cooler blower	Clear of obstructions

Oil cooler	Condition and no leaks

	Oil filter bypass indicator	Check in

	Oil cooler access door	Secured

	Aft fairing	Secured

	Baggage compartment	Cargo secure, door secure

	Left exhaust cover	Removed

Port engine bay

Engine	Condition, security of attachments

Engine mounts	Condition and security

Exhaust stack and clamps	Condition and security

Hoses and tubing	Chafing, condition and security

Pneumatic Lines	Condition and security

Linear actuator	Condition and security

Engine anti-ice valve	Condition and security

Tail rotor driveshaft	Condition of splines, couplings and freedom of movement

Fuel System check

Fuel boost pump CBs	Out

Battery switch	On

Fuel valve	Off

Fuel drain button Stbd fuselage	Press button, take sample, release

Fuel valve	On

Starboard engine cover	Open

Fuel CBs	In

Caution LT CB	In

Airframe fuel filter	Press test button, check A/F FUEL FILTER caution light On, release check light out

Caution LT CB	Out

Battery switch	Off

	Engine cowling	Secured

	Air induction cowling	Secured

Transmission mounts	Condition and security

Isolation mount	Condition

	Access door	Secured

	Cabin roof, transmission fairing

and engine air inlet area	Cleared of all debris

	Engine Plenum chamber	Check clear

Rotor head reservoirs	Visible oil levels (if fitted)

Main rotor hub and yoke	Condition

Main rotor blade doublers and skin	Condition

Pitch horn trunnion bearing	Wear and security

Main rotor pitch links	Condition and security

Swashplate assembly	Condition, security of attached controls, and boot

Control linkages to swashplate	Condition, security of attachment bolts and locking hardware

	Forward fairing and access door	Secured

	Cabin doors	Condition and security

	Windows	Condition and security

	Landing gear	Condition

	Forward and aft crosstube fairings	Secure, condition and aligned

	Left static port	Clear

	Exterior surfaces	Condition

	Windshield	Condition and cleanliness

Battery	Connected

Battery vent lines	Condition and security

Battery access door	Secured

	Pitot tube	Cover removed, clear of obstructions

	External power door	Condition and security

Landing light glass	Condition

	Antennas	Condition and security

	External power	Connect for APU start

Interior

Interior	Clean and tidy

Fire extinguisher	Secure, in check

First Aid kit	On rear shelf, in check

Seat belts	Condition

Life jackets	On rear shelf, in check

Headsets	Sufficient and serviceable

Briefing cards	On display

Airsickness bags	Sufficient

Maps	As required

Technical log	Completed

Operations Manual	Stowed if required

Aircraft documents	Stowed if required

Pre start

Pedals	Adjust

Harness	Secure

Doors	Shut

Control frictions	Release, Check full and free movement

Cyclic friction	On

Throttle	Open fully, close to shut off

Landing lights	Off

Engine anti-ice	Off

Hydraulics control boost switch	On

Radios and navaids	Off

Fuel valve	On and guarded

Flight Instruments	Check

Altimeter	Set QNH or QFE as required

Engine instruments	Check

GPS	On

E2 Compass	Serviceable

OAT Gauge	Serviceable

Note:  Ensure INST LT switch is OFF for day operations

Generator switch	Off

Battery Master 	On (Off is external power used)

Circuit breakers	IGN, START & CAUTION, In

CWP engine out light	On

Engine out audio	On

CWP test / Warning lights	Press to test, captions illuminated

Collective Lever	Raise off stop, check low RPM horn

Low rpm warning	CWP caption illuminated, audio warning

Collective Lever	Down, friction on

Booster pump CBs	Check each in turn, check pressure, 		leave one In for start

Caution CB	Out

Floats	CB as required, Master switch off

Rotor brake	Off (if fitted)

Cabin heat	Off

Anti-Collision light	On

Engine Starting

Collective lever	Down

Throttle	Closed

Governor	Beep fully aft

Rotors	Area clear

Starter	Engage

Engine oil pressure	Indicating

TOT	Below 150ºC

Throttle	Open to ground idle at 12-15% N1

Caution:  A start should not be attempted at N1 below 12%

TOT	Monitor to avoid hot start.  Abort start if temperature is likely to exceed 793C to 927ºC for 10 seconds.

Rotors	Turning by 25% N1

Starter	Release at 58% N1

N1	Accelerate through 50-60% Nr, flight idle at 62-67%N1

Engine oil pressure	Check

Transmission oil pressure	Check

Caution:   If the engine has been shut down for more than 15 minutes, stabilise at ground idle for one minute before increasing power.

Fuel pumps	Both Out

External power	Remove, Battery On

Throttle 	Open to 70% N1

Generator switch	On

Loadmeter	Less than 70%

DI / AH switch	On

Position lights	As required

Caution CB	In

Throttle	Open fully; Do not exceed 40% Torque or 625ºC max TOT during acceleration.

N2 governor	Check range 95% to 100% N2, set 100% N2

Radios/navaids	On

Engine anti-ice	On, check TOT rise.  If OAT below 5c leave On , otherwise Off

Control frictions	Off

Hydraulics switch	Off, check cyclic and collective for movement

Hydraulics switch	On

Control frictions	On

Throttle	Retard to ground idle, check N1 falls to 62%-67% within 3 seconds. Flight idle should be no lower than 62%

# Pre Take off

Fuel pumps	Both In

Floats	As required

CBs	As required

External lights	As required

Pitot heat	As required

CWP	Press to test

Engine instruments	Within limits

Flight instruments	Set DI & cage AH

Altimeter	QNH or QFE as required

Fuel	Sufficient

INST LT switch	As desired

Generator load	Below 0.7

Radios	Correct frequencies

Navaids	Correct frequencies, identified (if required)

Hydraulics	On

Anti icing	As required

Harnesses	Secure

Doors	Closed

After Take Off / Pre-Landing Checks

Controls	Normal response

Nr/N2	100%

Engine Instruments	Normal, within limits

T’s and P’s	Normal, within limits

CWP	No warning lights illuminated

CB’s/switches	As required

Outbound / En- route / Inbound Checks

Fuel	Sufficient, including reserve

Radio	Correct frequency, as required

Engine	T’s & P’s normal, within limits

Engine anti-ice	On in visible moisture when temperature is below 5c

Throttle	Fully open

Direction	As required & DI synchronised with E2

Altimeter	QFE/QNH, Regional QNH as required

Over water flight

Float power CB	In

Float Manual Arm switch	Guard open, switch On

Float Armed light	On

Caution:  At altitudes above 500 feet and at IAS above 70 mph the manual  system should be deactivated by positioning the Float Manual Arm switch to Off and guarded

Re-arm prior to landing

Shut down

Collective	Fully down, friction On

Cyclic	Centred, friction On

Pedals	Neutral

Throttle	Close to flight idle.

Stabilise for 2 minutes.

Then close, ensure engine wind down

Radio/Navaids 	Off

(leave one radio On until TOT stabilised, then off)

AH/DI switch	Off

Pitot heat	Off

Defog blower 	Off

Generator	Off

Floats	Disarmed, CB out

Fuel booster pump CBs	Out

CB’s	IGN, Start & Caution Out

Rotor brake	On between 30% & 38% Nr (if fitted)

Battery	Off

Caution CB	In (Battery On warning)

Anti-collision light	On (Battery On warning)

## Intentionally Blank

CWP Captions

Trans Temp

Main transmission oil temperature is at or above red line

Reduce power

Transmission oil pressure	Check

Land as soon as possible

Trans Press

Main transmission pressure is below minimum

Land as soon as possible

Fuel Pump

One or both fuel boost pumps is inoperative. Descend to below 6000 feet pressure altitude if flight permits.

Land as soon as practical

Warning:   Operation with both fuel boost pumps inoperative is not authorised.  Due to possible fuel sloshing in unusual attitudes or out of trim conditions and one or both fuel boost pumps inoperative, the unusable fuel is ten gallons (US)

The engine will operate without boost pump pressure under 6000 feet pressure altitude and one boost pump will supply sufficient fuel for normal engine operations under all conditions of power

Fuel Filter

Engine fuel filter clogged

Land as soon as practical

Clean before next flight

A/F Fuel Filter

Airframe fuel filter clogged

Land as soon as practical

Clean before next flight

Fuel Low

Approximately 20 gallons of fuel remaining.

Land as soon as practical

Eng Out

When this system is activated an intermittent audio signal is produced and the ENG OUT light is ON.

Engine power failure

Engine out audio on

Enter autorotation

Carry out Engine Failure in Flight drills

Eng Chip

Metallic particles in engine oil

Land as soon as possible

T/R Chip

Metallic particles in tail rotor gear box oil

Land as soon as possible

Trans Chip

Metallic particles in transmission oil

Land as soon as possible

### Batt Temp

Battery case temperature has reached 130º F or higher.

Battery switch	Off

Battery switch	On when light extinguishes

Batt Hot

Battery case temperature has reached 140º F or higher.

Battery switch	Off

Land as soon as possible

Rotor Low RPM

When this system is activated the rotor low rpm light is ON and a steady audio signal is produced, only when the collective pitch is off the down stop. Rotor RMP is below normal.

Collective lever	Reduce pitch

Throttle	Full open

Engine Failure in Flight

Enter autorotation

Collective lever	Fully down, adjust to maintain rotor rpm 90 to 107%

IAS	58 to 69 mph  (50 to 60 Knots)

Mayday call

Throttle	Closed

Fuel valve	Off

Fuel booster pump Cbs	Out

Carry out EOL

Engine Failure in the Hover

Left yaw

Stop yaw and drift

Cushion touchdown with lever

Complete shutdown checks

Engine Fire During Starting or Shutdown

An engine fire during start could be caused by an overloading of fuel in the combustion chamber and a delayed ignition of the fuel, resulting in flame emanating from the tailpipe. To extinguish fire, proceed as follows:

Starter	Continue to motor the engine.

Throttle	Full Closed.

Fuel Valve Switch	Off

Ign eng CB	Out

Boost pump CBs	Out

Battery switch	Off

Vacate helicopter

Use hand fire extinguisher as appropriate

Engine Fire During Flight

Enter autorotation.

Throttle	Close

Fuel valve switch	Off

Boost pump Cbs	Out

Mayday call

Carry out Engine failure in Forward Flight drills

Battery switch	Off

Generator switch	Off

Tail rotor Control failure

Stuck pedals

Adjust power to minimise excessive yawing

Perform zero speed or running landing as required

Loss of Tail rotor thrust

Right yaw, no response with left pedal

Enter autorotation

Throttle Closed to flight idle

Maintain minimum of 58 mph (50 knots)

If possible land with wind 50-60º from the left

Perform EOL

Note:  The large ventral fin may permit controlled flight at low power levels and sufficient air speed when a suitable landing site is not available; however, the touch-down shall be accomplished with the throttle in the full closed position.

Fuel Control And/Or Governor Failure

Engine fuel control and/or governor failure will be apparent by a change of power or RPM. There is no manual fuel control on the engine.

Engine Underspeed

Lower lever to control rpm, enter autorotation and carry out and EOL if necessary

Land as soon as possible

Engine Overspeed

Control rpm with throttle

Land as soon as possible

Hydraulic System Failure

The first indication  of hydraulic boost failure will be an increase in the force required for control movement; feed-back forces will be noticed as well as rate limiting. Control motions will result in normal flight reactions in all respects, except for the increased force required for control movement. In the event of hydraulic power failure, proceed as follows:

Reduce airspeed to 70 to 80 MPH IAS (61 to 69 KIAS).

Hydraulic switch	On

Hyd boost CB	Out,  If power is not restored In

Hydraulic switch	Off

Land as soon as practical

A run-on landing at 12 to 17 MPH (10 to 15 knots) is recommended.

Electrical Power Failure

Electrical power for flight is furnished by the starter which is utilized as a generator after the start has been accomplished. Evidence of main generator failure will be provided by observing loadmeter load. There is no provision for stand-by operation in the event of generator failure. Necessary power can be furnished by the battery for short periods of time.

In case of generator failure:

Cycle generator switch

If power is not restored	Off

All electrical equipment	Off (to conserve battery).

Required electrical equipment	On, only as needed.

WARNING: Reduce altitude to below 6000 feet. No booster pumps

Helicopter Fuel System Failure

Helicopter fuel system failure will be caused by a malfunction of one or both of the fuel boost pumps and will be evident by the illumination of the FUEL PUMP caution light. The engine will operate without boost pump pressure under 6000 ft altitude conditions and one pump will supply sufficient fuel for normal engine operation under all conditions of power and altitude.

WARNING:  Operation with both fuel boost pumps inoperative is not approved. Due to possible fuel sloshing in unusual attitudes or out of trim conditions and one or both fuel boost pumps inoperative, the unusable fuel is ten gallons.

Engine Air Start

When cause of the engine failure is believed to be mechanical, do not attempt a restart.

Nr	90 to 107%.

IAS	58 to 69 mph (50 to 60 knots).

Generator	Off

Proceed with normal start

Caution:   Do not attempt air start above 12,000 feet. (TOT rises too fast to control)

### Limitations

Max all up weight	3200lbs

Vne	150 mph at MSL

Altitude Limitations:

AUW above 3000 lbs	13500 ft DA

AUW below 3000 lbs	20000 ft DA

Rotor limits:

Power on	97 - 100%

Power off	90 - 107%

Fuel pressure:	4 - 30 psi

Oil Pressure

below 78.8% N1	50 - 90 psi

78.5 - 94.2% N1	90 - 115 psi

above 94.2% N1	115 - 130 psi

Oil Temperature	107ºC max

Engine power

Take off	100% Tq (5 minutes)

Transient	110%  Tq (5 seconds)

Max cont	85% Tq

TOT limits

Take off	793ºC (5 minutes)

Max Cont	737ºC

Power transient	793 to 843ºC (6 seconds)

Start up	793 tp 927ºC (10 secs)

Trans oil Pressure	30 psi Min

30 to 50 psi Continuous

70 psi Maximum

Trans Oil Temp	15 to 110ºC Cont

110ºC Maximum

Bell 206 Limited power margins for training

Maximum power available is N1 at the 5  minute take off power, ie. 100% torque or 793º C TOT, whichever is limiting or 104 % N1.

Take off	Check N1 in a 2 ft hover.

Power margin	Technique

< 1%		Running take off

> 1%		Cushion creep

>2 %		Normal shallow climb

>3 %		Normal steep climb

>4 %		Towering take off

>5 %		Vertical OGE take off

Landing	Check N1 at 47 Knots (55 mph) in straight and level , balanced flight.

Power margin	Technique

< 4 %		Running landing

4 - 6 %		Zero speed

> 7%		Normal approach to low hover

> 8%		Steep approach

>10 %		Vertical OGE approach